MicroprocessorS PROJECT REPORT

Radar System

Using 8051 Microcontroller

Ultrasonic Proximity Detection System

| SUBMITTED BY | Marwan Hossam — 231005406 Ahmed ElSayed — 231006905 Ahmed Gamal — 231004245 |
| --- | --- |

June 2026

# 1. Abstract

This project presents the design and implementation of an ultrasonic distance measurement system — referred to as a Radar System — using the AT89S52 microcontroller. The system employs an HC-SR04 ultrasonic sensor to measure distances ranging from 2 cm to 200 cm, displays real-time readings on a 16×2 LCD, and provides visual and audible feedback through LEDs and a buzzer. The system is programmed in Assembly language using the ASEM-51 assembler and flashed onto the microcontroller via AVRDUDE with an Arduino UNO acting as an ISP programmer.

# 2. Introduction

Distance measurement is fundamental in many engineering domains, including robotics, industrial automation, parking assistance, and security systems. Ultrasonic sensors provide a non-contact, cost-effective approach to distance measurement. This project demonstrates a complete embedded system that measures distance and provides multi-level feedback based on predefined thresholds.

## 2.1 Objectives

- Measure distance from 2 cm to 200 cm using the HC-SR04 ultrasonic sensor

- Display real-time distance readings on a 16×2 LCD

- Provide visual indication via three LEDs (Green, Yellow, Red) based on distance ranges

- Activate an audible buzzer for critical distances (less than 10 cm)

- Implement a complete embedded system using the AT89S52 microcontroller

# 3. System Components

## 3.1 Hardware Components

| Component | Specification | Qty |
| --- | --- | --- |
| AT89S52 Microcontroller | 8-bit, 8KB Flash, 11MHz Crystal | 1 |
| HC-SR04 Ultrasonic Sensor | 2cm–400cm range, 40kHz | 1 |
| 16×2 LCD Display | HD44780 Compatible | 1 |
| Green LED | 5mm | 1 |
| Yellow LED | 5mm | 1 |
| Red LED | 5mm | 1 |
| Buzzer | 5V Active | 1 |
| Resistors | 220Ω (LEDs), 10kΩ (pull-ups) | 10 |
| Potentiometer | 10kΩ – LCD Contrast | 1 |
| Power Supply | 5V DC | 1 |
| Arduino UNO | ISP Programmer | 1 |

## 3.2 Software Tools

| Tool | Version | Purpose |
| --- | --- | --- |
| ASEM-51 | V1.3 | Assembly language compiler |
| AVRDUDE | 8.0.0 | Microcontroller programmer |
| ArduinoISP | — | Programmer firmware for Arduino UNO |

# 4. System Architecture

## 4.1 Pin Connections

The AT89S52 interfaces with all peripherals as described in the tables below.

## 4.1.1  HC-SR04 Ultrasonic Sensor

| HC-SR04 Pin | Connected To |
| --- | --- |
| VCC | +5V |
| GND | GND |
| TRIG | P1.0 |
| ECHO | P1.1 |

## 4.1.2  LCD Display (4-bit Mode)

| LCD Pin | Function | Connected To |
| --- | --- | --- |
| 1 | VSS | GND |
| 2 | VDD | +5V |
| 3 | V0 (Contrast) | Potentiometer / GND |
| 4 | RS | P2.4 |
| 5 | RW | GND |
| 6 | EN | P2.5 |
| 11 | D4 | P2.0 |
| 12 | D5 | P2.1 |
| 13 | D6 | P2.2 |
| 14 | D7 | P2.3 |
| 15 | LED+ | +5V (via 220Ω) |
| 16 | LED- | GND |

## 4.1.3  LEDs and Buzzer

| Component | Connected To | Active State |
| --- | --- | --- |
| Green LED (Anode) | +5V via 220Ω → Cathode to P3.3 | LOW = ON |
| Yellow LED (Anode) | +5V via 220Ω → Cathode to P3.4 | LOW = ON |
| Red LED (Anode) | +5V via 220Ω → Cathode to P3.5 | LOW = ON |
| Buzzer (+) | +5V → (–) to P2.6 | LOW = ON |

# 5. Working Principle

## 5.1 Ultrasonic Distance Measurement

The HC-SR04 sensor operates on echo-timing principles:

- A 10 µs HIGH pulse is sent to the TRIG pin

- The sensor emits 8 cycles of 40 kHz ultrasonic sound

- Sound waves reflect off objects and return to the sensor

- The ECHO pin goes HIGH when sound is emitted and LOW when the echo is received

- The ECHO pulse width is directly proportional to the measured distance

## 5.2 Distance Calculation Formula

The microcontroller's Timer 0 (Mode 1, 16-bit) measures the ECHO pulse width. The timer value is then divided by 58 to obtain distance in centimeters:

Distance (cm) = Echo Pulse Width (µs) ÷ 58

Where: Speed of Sound = 343 m/s = 0.0343 cm/µs, divided by 2 for the round trip.

## 5.3 Feedback Logic

| Distance Range | Green LED | Yellow LED | Red LED | Buzzer | LCD Warning |
| --- | --- | --- | --- | --- | --- |
| > 30 cm | ON | OFF | OFF | OFF | — |
| 20 – 30 cm | OFF | ON | OFF | OFF | — |
| 10 – 20 cm | OFF | OFF | ON | OFF | — |
| < 10 cm | OFF | OFF | ON | ON | <10cm! |

## 5.4 Hardware Implementation

# 6. Software Implementation

## 6.1 Program Schematic

The program follows a straightforward sequential loop:

- Timer 0 initialization and LCD initialization on startup

- Display static header on LCD

- Enter main loop: measure distance → update display → control LEDs → control buzzer → 200 ms delay

## 6.2 Key Functions

| Function | Description |
| --- | --- |
| MEASURE_DISTANCE | Triggers HC-SR04 and measures ECHO pulse width via Timer 0 |
| DIV_58 | Converts raw timer value to distance in centimeters |
| UPDATE_DISPLAY | Updates LCD only when distance value changes |
| CONTROL_LEDS | Sets LED states based on distance thresholds |
| CONTROL_BUZZER | Activates buzzer when distance < 10 cm |
| LCD_INIT | Initializes LCD in 4-bit mode with required timing |
| LCD_CMD / LCD_DATA | Sends commands or character data to LCD |
| LCD_STRING | Displays a null-terminated string from code memory |

## 6.3 Timing Calculations (11 MHz Crystal)

- Machine cycle = 12 × clock cycle = 12 / 11 MHz ≈ 1.09 µs

- Delay loops calibrated for 1 ms, 5 ms, 10 ms, 50 ms, 100 ms, and 200 ms

- 10 µs delay used for HC-SR04 trigger pulse

# 7. Testing and Results

| Test Case | Expected Result | Status |
| --- | --- | --- |
| Distance > 30 cm | Green LED ON | Pass |
| Distance 20–30 cm | Yellow LED ON | Pass |
| Distance 10–20 cm | Red LED ON | Pass |
| Distance < 10 cm | Red LED ON + Buzzer + LCD Warning | Pass |
| LCD Display | Correct real-time distance reading | Pass |
| Measurement Range | 2 cm to 200 cm | Pass |

# 8. Challenges and Solutions

## 8.1 LCD Initialization

Challenge: LCD displaying garbage characters or blank screen.

Solution: Grounded the RW pin for write-only operation.

## 8.2 Timing Accuracy

Challenge: Inconsistent distance readings due to timing errors.

Solution: Calibrated delay loops for the exact 11 MHz crystal, applied Distance = Time / 58 formula, and added timer overflow protection.

## 8.3 LED Polarity

Challenge: LEDs toggling with incorrect logic (wrong polarity).

Solution: Identified common-anode configuration (LED+ to +5V), inverted control logic (CLR = ON, SETB = OFF), and updated the code accordingly.

# 9. Applications

This system can be applied in a variety of real-world scenarios:

- Robotics — Obstacle detection and avoidance

- Parking Assistance — Vehicle reverse guidance

- Industrial Automation — Liquid level monitoring

- Security Systems — Motion and intrusion detection

- Smart Homes — Automatic doors and occupancy-based lighting

- Education — Embedded systems demonstration and teaching tool

# 10. Conclusion

The Radar System project successfully demonstrates the implementation of a complete embedded system using the AT89S52 microcontroller. The system reliably measures distances from 2 cm to 200 cm, displays readings on a 16×2 LCD, and delivers multi-level visual and audible feedback.

All project objectives were achieved:

- Real-time distance measurement with up to 2% error at long range

- LCD display with live updates

- Three-zone LED indication (Green / Yellow / Red)

- Buzzer alert for critical proximity (< 10 cm)

The system is reliable, cost-effective, and extensible for various real-world applications. It serves as a solid foundation for understanding microcontroller programming, sensor interfacing, and embedded system design.

# 11. Bill of Materials

| Component | Part / Spec | Cost(egp) |
| --- | --- | --- |
| AT89S52 Microcontroller | AT89S52-24PU | 100 |
| HC-SR04 Sensor | HC-SR04 | 70 |
| 16×2 LCD Display | LM016L | 80 |
| LEDs (3 pcs) | 5mm Green / Yellow / Red | 2.5 |
| Buzzer | 5V Active Buzzer | 5 |
| Resistors (10 pcs) | 220Ω & 10kΩ | 2.5 |
| Crystal Oscillator | 11.0592 MHz | 5 |
| Documentation Report |  | 30 |
| Total |  | 300 |

# 12. References

- AT89S52 Datasheet — Atmel / Microchip Technology

- HC-SR04 Ultrasonic Sensor Datasheet

- HD44780 LCD Controller Datasheet

- ASEM-51 Assembler Documentation

- AVRDUDE User Manual